GOVERNMENT

OF SPAIN

RAIL ACCIDENT

INVESTIGATION COMMISSION

UNDERSECRETARIAT

DEVELOPMENT

MINISTRY

on the investigation of rail accidents

and incidents on the

General Interest Rail Network

Rail Accident Investigation Commission

September 2009

CONTENTS

CONTENTS

1.	INTRODUCTION	3

Subject matter and scope	3

Investigation of events occurring during 2008	4

The rationale of accident investigation	6

The introduction of Safety Directive 49/2004/EC	7

2.	THE ORGANISATION OF ACCIDENT INVESTIGATION	8

Organisational Structure of the Rail Accident Investigation Commission	8

3.	THE INVESTIGATION PROCESS	9

The process of investigating events	9

4.	INVESTIGATION OF ACCIDENTS AND INCIDENTS	10

Categorisation of events	10

Events investigated on the General Interest Rail Network	14

Analysis by category	16

1.	Serious accidents	16

1.1	Accidents due to collision	16

2.	Accidents	18

2.1	Persons struck by rolling stock	18

2.2	Collisions	21

2.3	Derailments	21

2.4.	Level crossing accidents	23

2.5	Accidents due to fire	27

2.6	Other accidents	27

3.	Incidents	27

5.	CONCLUSIONS	30

Causes of the events investigated	30

6.	COMPARISON	31

7.	REPORTS AND RECOMMENDATIONS ISSUED	33

Recommendations contained in the accident and incident reports	33

# 1.	INTRODUCTION

## Subject matter and scope

For the fourth consecutive year we present our annual report which, for the first time, contains information not only on the technical investigations undertaken and concluded in relation to the rail accidents which have occurred on the General Interest Rail Network during 2008, but also on a number of incidents which, by their nature, were the subject of an investigation.

The entry into force of the Reglamento sobre seguridad en la circulación de la Red Ferroviaria de Interés General [Regulations on Traffic Safety in the General Interest Rail Network] (Royal Decree 810/2007 of 22 June) in the last quarter of 2007 marked a turning point in the investigation of accidents on the networks managed by the Administrador de Infraestructuras Ferroviarias (ADIF) [Rail Infrastructure Manager] and by Ferrocarriles Españoles de Vía Estrecha (FEVE) [Spanish Narrow Gauge Railways], since it adopted the narrow definition of a serious accident contained in Directive 49/2004/EC – more restrictive than that laid down in the previous legislation, the Reglamento del Sector Ferroviario [Railway Sector Regulations] (Royal Decree 2387/2004 of 30 December) and defined the powers of the regulatory body to carry out an investigation of any rail accident or incident where it was considered appropriate on the grounds that it could contribute to improving rail safety.

Accordingly, during its first full year of investigating rail accidents, the Rail Accident Investigation Commission adopted a new approach by undertaking investigations of events (accidents and incidents) based on a rationale which had more to do with specific railway aspects than the kind of events which had been investigated in the past, where the causes were attributed to the actions of third parties, which normally meant the imprudent behaviour of the victims.

This report therefore contains data obtained from investigations undertaken in 2008 (all of which have been completed) into those rail incidents which the Commission considered appropriate to examine, out of all the events notified by the infrastructure administrators (Adif and Feve). They all occurred between 1 January and 31 December 2008. At the end of the report there is a summary of all the technical reports and recommendations formulated in relation to these occurrences.

However, the Commission has continued to investigate all events which were classed as serious under the old rules, the Reglamento de la Ley del Sector Ferroviario [Regulations of the Law of the Rail Sector] (Royal Decree 2387/2004) and, as indicated earlier, has also investigated other occurrences not classed as accidents, where this was considered relevant to the improvement of rail safety. These investigations mainly relate to incidents involving near misses and derailments, as established in CIAF Circular 2/2008.

## Investigation of events occurring during 2008

A total of 57 events on the General Interest Rail Network were investigated in 2008, and were categorised as follows:

|  | Serious accidents and incidents |  |  |  | Incidents |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Person struck | Collision | Derailment | Level crossing * | At danger signal passed / near miss | Total |
| Adif | 33 | 2 | 2 | 14 | 2 | 53 |
| Feve | 2 | 1 | 0 | 0 | 1 | 4 |
| Total | 35 | 3 | 2 | 14 | 3 | 57 |

*Includes persons struck on level crossings.

Events investigated in 2008

At danger signal passed /

near miss

Level

crossing

Derailment

Collision

Person struck

| Person struck | Collision | Derailment | Level crossing | At danger signal passed / near miss |
| --- | --- | --- | --- | --- |

Adif 	  Feve

Of the events investigated, the highest proportion (61.40%) relates to persons being struck by rolling stock, followed by those occurring at level crossings (24.56%).

Person struck      Collision     Derailment      Level crossing      At danger signal passed / near miss

|  | Serious accident | Accident |  |  |  |  | Incident |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Month | Collision | Person struck | Collision | Derailment |  | Level crossing | At danger signal passed / near miss | Total |
| January |  | 2 |  |  |  | 1 |  | 3 |
| February | 1 | 1 |  |  |  |  |  | 2 |
| March |  | 1 |  |  |  | 1 |  | 2 |
| April |  | 1 |  |  |  | 1 |  | 2 |
| May |  | 6 |  |  |  | 3 |  | 9 |
| June |  | 3 |  |  |  | 2 |  | 5 |
| July |  | 3 |  |  | 1 |  | 1 | 5 |
| August |  | 1 |  |  |  |  |  | 1 |
| September | 1 | 4 | 1 |  |  |  |  | 6 |
| October |  | 5 |  |  | 1 | 2 | 1 | 9 |
| November |  | 7 |  |  |  | 2 | 1 | 10 |
| December |  | 1 |  |  |  | 2 |  | 3 |
| Total | 2 | 35 | 1 |  | 2 | 14 | 3 | 57 |

The events investigated show a relatively consistent time distribution over the year, with a maximum in November and a minimum in August. 85.96% of these events were the result of actions by persons unconnected with the railways.

Monthly distribution of the accidents and incidents investigated

January    February     March        April        May           June 	July         August   September  October  November  December

Person struck               Collision 	     Derailment 	    Level crossing       At danger signal passed/near miss

## The rationale of accident investigation

The purpose of investigating rail accidents is to ascertain their causes and the circumstances in which they occurred, with a view to preventing future accidents by making the appropriate recommendations to reduce risks in rail transport.

The investigation will not under any circumstances be concerned with determining fault or liability and will be independent of any judicial investigation, as stated in Directive 49/2004/EC and specified in Article 21(6) of the Traffic Safety Regulations.

When rail incidents are investigated a technical report is then produced, containing all the data relating to the occurrence, the investigations carried out, the conclusions and, where appropriate, the recommendations made.

## The introduction of Safety Directive 49/2004/EC

Safety Directive 49/204/EC defined the principles for the management and regulation of rail safety for the common European railway area and, amongst other things, specified that each Member State should create appropriate bodies with responsibility for rail safety and for investigating rail accidents.

In Spain the legislative transition process which began with the promulgation of the Ley del Sector Ferroviario [Law on the Railway Sector] (Law 39/2003 of 17 November, Official State Gazette No 276 of 18 November) and its implementing regulation (Royal Decree 2387/2004 of 30 December, Official State Gazette No 315 of 31 December) culminated in the approval of the Reglamento sobre seguridad en la circulación de la red Ferroviaria de Interés General [Regulations on Traffic Safety in the General Interest Rail Network] (Royal Decree 810/2007 of 22 June, published in Official State Gazette No 162 of 7 July), which transposed the Directive on safety on the Community's railways and repealed the provisions of the earlier accident investigation rules contained in the Railway Sector Regulations.

The entry into force of these new Regulations gave rise, amongst other things, to the creation of a new investigative body - the Rail Accident Investigation Commission (CIAF) - to replace the Dirección General de Ferrocarriles [Railways Directorate] which from now on would confine itself to rail safety regulatory and inspection functions.

The CIAF began operating in December 2007, and has been operational throughout 2008, handling all the investigative functions formerly assumed by the Railways Directorate.

# 2.	THE ORGANISATION OF ACCIDENT INVESTIGATION

## Organisational Structure of the Rail Accident Investigation Commission

The Rail Accident Investigation Commission (CIAF) is a specialist collegiate body subordinate to the Development Ministry, and was constituted on 11 December 2007. It consists of a President, a Plenary and a Secretariat. The Plenary consists of the President and five members.

In addition to this formal structure, there are technical investigators attached to the Secretariat, who are responsible for conducting investigations in collaboration with a team of representatives from the companies involved in the event being investigated, and subsequently for producing the relevant reports.

The CIAF also has a territorial network of technical staff at its disposal, and can call on various technical specialists employed on a contract basis.

The following chart shows the structure in schematic form:

THE ORGANISATION OF RAIL ACCIDENT INVESTIGATION IN THE C.I.A.F.

# 3.	THE INVESTIGATION PROCESS

## The process of investigating events

Following an initial period during which it provisionally agreed to keep in place the Circular Orders which had governed the investigative process up to that time, the Rail Accident Investigation Commission drafted and approved the following Circular Orders in the first months of its operation:

Circular Order 1/2008 Pautas para la investigación técnica de los accidentes ferroviarios que competen a la Comisión de Investigación de accidentes ferroviarios [Guidelines for the investigation of rail accidents within the responsibility of the Rail Accident Investigation Commission], which entered into force on 16 March 2008. It established guidelines for the technical investigation of rail accidents by the CIAF investigation team.

Circular Order 2/2008 Comunicación de los incidentes ferroviarios y los suicidios [Notification of rail incidents and suicides] and its subsequent amendment, which entered into force on 10 June 2008. It established the guidelines to be followed by the infrastructure administrators when notifying the CIAF about those rail occurrences which are not classed as accidents.

In addition to these rules, the Commission drafted a document entitled Procedimiento de investigación técnica de los accidentes e incidentes ferroviarios [Procedures for the investigation of rail accidents and incidents] the purpose of which is to establish basic guidelines for the investigation of rail accidents and incidents by the CIAF and to help define the process, in addition to establishing the relationships between the various bodies taking part in the investigation.

The text, approved on 10 November 2008, revokes Circular Order 1/2008 except in the case of accidents whose consequences consist exclusively of injuries to persons caused by rail vehicles in motion (persons struck, falls, etc.), for which another specific procedure was developed, the Procedimiento para la investigación técnica de accidentes de personas producidas por material rodante en movimiento [Procedure for the technical investigation of accidents to persons caused by rolling stock in  motion] which entered into force on 31 July 2009.

# 4.	INVESTIGATION OF ACCIDENTS AND INCIDENTS

## Categorisation of events

As mentioned earlier, during 2008 the criterion for the investigation of rail accidents and incidents was based on the definitions of an accident, serious accident and incident and the terms of Article 21.3 of the Reglamento sobre seguridad en la circulación de la Red Ferroviaria de Interés General [Regulations on Traffic Safety in the General Interest Rail Network] (Royal Decree 810/2007 of 22 June), namely:

‘Accident: an unwanted or unintended sudden event or a specific chain of such events which have harmful consequences; accidents are divided into the following categories: collisions, derailments, level-crossing accidents, accidents to persons caused by rolling stock in motion, fires and others.’

‘Serious accident: any train collision or derailment of trains, resulting in the death of at least one person or serious injuries to five or more persons or extensive damage to rolling stock, the infrastructure or the environment, and any other similar accident with an obvious impact on railway safety regulation or the management of safety; “extensive damage” means damage that can immediately be assessed by the investigating body to cost at least EUR 2 million in total.’

‘Incident: Any event other than an accident or serious accident, associated with the use and operation of trains or rolling stock and affecting traffic safety.’

Article 21.3:  ‘In reaching a decision to investigate rail accidents not classed as serious, non-serious incidents and rail incidents, the Rail Accident Investigation Commission shall assess the following circumstances:

a)	   the importance of the accident or incident;

b)	whether it forms part of a series of accidents or incidents with repercussions for the system as a whole;

c)	the repercussions for the safety of rail transport

d)	requests from the Rail Infrastructure Administrator, the rail companies or the Development Minister.’

Investigations carried out

74 events were reported to the CIAF during 2008, of which 57 were investigated: 53 were accidents and incidents which occurred on the rail network administered by Adif and 4 on the network administered by Feve.

The Table on the following page summarises the characteristic details of each of these events, in the order of their case number. All investigations commenced in 2008 have already been completed.

4.  INVESTIGATION OF ACCIDENTS AND INCIDENTS

| NET WORK | OPERATOR | CASE Nº | DATE | MUNICIPALITY | PROVINCE | LINE | K.P. | FATALITIES | SERIOUS INJURIES | CATEGORY OF EVENT | TYPE OF EVENT | VEHICLE / PEDESTRIAN | LOCATION | STATUS | RECOMMENDATIONS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADIF | RENFE | 001/08 | 10/01/2008 | Alcala de Córdoba | Córdoba | 400 Alcázar de San Juan-Cádiz | 429,825 | 1 | 0 | ACCIDENT | LEVEL CROSSING | BICYCLE | STATION | CLOSED | 1 |
| ADIF | RENFE | 002/08 | 26/01/2008 | A Rua Petin | Orense | 800 La Coruña-León | 310,412 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 0 |
| ADIF | RENFE | 003/08 | 22/02/2008 | Salbio | Álava | 700 Casetas-Intermodal Abando Indalecio Prieto | 218,654 | 1 | 0 | SERIOUS ACCIDENT | COLLISION | MOTOR VEHICLE | HALT | CLOSED | 0 |
| ADIF | RENFE | 004/08 | 14/03/2008 | Llinars del Vallés | Barcelona | 270 Portbou-Cerbere-Barcelona-Sant Andreu Comtal | 148,000 | 1 | 0 | ACCIDENT | LEVEL CROSSING | MOTORCYCLE | STATION | CLOSED | 0 |
| ADIF | RENFE | 006/08 | 21/03/2008 | Sant Adriá de Besós | Barcelona | 276 Maçanet-Massanes-Barcelona-Sagrera | 3,286 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 1 |
| ADIF | RENFE | 009/08 | 16/04/2008 | Sant Feliu de Llobregat | Barcelona | 240 San Vicente de Calders-L´Hospitalet de Llobregat | 88,906 | 1 | 0 | ACCIDENT | LEVEL CROSSING | PERSON | STATION | CLOSED | 0 |
| ADIF | RENFE | 0010/08 | 22/04/2008 | Zorrotza | Viscaya | 720 Intermodal Abando Indalecio Prieto-Santurtzi | 5,348 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | STATION | CLOSED | 0 |
| ADIF | RENFE | 0012/08 | 06/05/2008 | Fabara | Zaragoza | 210 Miraflores-Tarragona | 470,754 | 1 | 0 | ACCIDENT | PERSON STRUCK | PASSENGER | STATION | CLOSED | 1 |
| ADIF | RENFE | 0013/08 | 09/05/2008 | Cetina | Zaragoza | 200 Madrid-Barcelona | 214,220 | 2 | 0 | ACCIDENT | LEVEL CROSSING | MOTOR VEHICLE | TRACK | CLOSED | 1 |
| ADIF | RENFE | 0014/08 | 13/05/2008 | Alegia | Guipúzcoa | 100 Madrid-Hendaya | 594,378 | 1 | 0 | ACCIDENT | PERSON STRUCK | STAFF | TRACK | CLOSED | 1 |
| ADIF | RENFE | 0015/08 | 13/05/2008 | Hernani | Guipúzcoa | 100 Madrid-Hendaya | 615,049 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | HALT | CLOSED | 1 |
| ADIF | RENFE | 0016/08 | 14/05/2008 | Villadepalos | León | 800 La Coruña-León | 263,566 | 1 | 0 | ACCIDENT | LEVEL CROSSING | PERSON | HALT | CLOSED | 0 |
| ADIF | RENFE | 0017/08 | 24/05/2008 | Villena | Alicante | 330 La Encina-Alicante | 396,675 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 2 |
| ADIF | RENFE | 0018/08 | 27/05/2008 | Santa Cruz de la Zarza | Toledo | 310 Aranjuez-Valencia | 39,950 | 1 | 0 | ACCIDENT | LEVEL CROSSING | BACKHOE EXCAVATOR | TRACK | CLOSED | 1 |
| ADIF | RENFE | 0019/08 | 24/05/2008 | Aznalcázar Pilas | Sevilla | 440 MajarabiqueEstación-Huelva Término | 24,900 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 1 |
| ADIF | RENFE | 0020/08 | 03/06/2008 | Figueres | Gerona | 270 Portbou-Cerbere-Barcelona-Sant Andreu Comtal | 247,215 | 1 | 0 | ACCIDENT | LEVEL CROSSING | PERSON | STATION | CLOSED | 0 |
| ADIF | RENFE | 0021/08 | 03/06/2008 | Lora del Río | Sevilla | 400 Alcázar de San Juan-Cádiz | 511,635 | 1 | 0 | ACCIDENT | LEVEL CROSSING | MOTOR VEHICLE | TRACK | CLOSED | 1 |
| ADIF | RENFE | 0022/08 | 06/01/2008 | Madrid | Madrid | 100 Madrid-Hendaya | 20,100 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 0 |
| ADIF | RENFE | 0023/08 | 05/02/2008 | Madrid | Madrid | 300 Madrid-Valencia | 13,100 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 1 |
| ADIF | RENFE | 0024/08 | 22/06/2008 | Tamarite-Altorricón | Huesca | 200 Madrid-Barcelona | 149,950 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 0 |
| ADIF | RENFE | 0026/08 | 24/06/2008 | Cabrera de Mar | Barcelona | 276 Maçanet-Massanes-Barcelona-Sagrera | 26,100 | 1 | 1 | ACCIDENT | PERSON STRUCK | PERSON | BRIDGE | CLOSED | 2 |
| ADIF | RENFE | 0028/08 | 30/06/2008 | Miranda de Ebro | Burgos | 700 Casetas-Intermodal Abando Indalecio Prieto | 146,950 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | STATION | CLOSED | 0 |
| ADIF | RENFE | 0029/08 | 05/07/2008 | Medina del Campo | Valladolid | 100 Madrid-Hendaya | 208,312 | 0 | 0 | ACCIDENT | DERAILMENT | TRAIN | STATION | CLOSED | 0 |
| ADIF | RENFE | 0030/08 | 16/07/2008 | Valencia | Valencia | 300 Madrid-Valencia | 109,900 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 0 |
| ADIF | RENFE | 0031/08 | 30/07/2008 | Andoaín | Guipúzcoa | 100 Madrid-Hendaya | 608,155 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | HALT | CLOSED | 0 |
| ADIF | RENFE | 0032/08 | 30/07/2008 | Málaga | Málaga | 436 Málaga- Fuengirola | 8,870 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 0 |
| ADIF | RENFE | 0037/08 | 29/08/2008 | Segur de Calafell | Tarragona | 200 Madrid-Barcelona | 624,800 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | HALT | CLOSED | 1 |
| ADIF | RENFE | 0038/08 | 01/09/2008 | Dueñas | Palencia | 100 Madrid-Hendaya | 279,000 | 1 | 0 | SERIOUS ACCIDENT | COLLISION | DUMPER | TRACK | CLOSED | 1 |
| ADIF | RENFE | 0039/08 | 11/05/2008 | Villarrobledo | Albacete | 300 Madrid-Valencia | 202,400 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 0 |

| NET WORK | OPERATOR | CASE Nº | DATE | MUNICIPALITY | PROVINCE | LINE | K.P. | FATALITIES | SERIOUS INJURIES | CATEGORY OF EVENT | TYPE OF EVENT | VEHICLE / PEDESTRIAN | LOCATION | STATUS | RECOMMENDATIONS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ADIF | RENFE | 0040/08 | 04/07/2008 | Sant Celoni | Barcelona | 270 Portbou-Cerbere –Barcelona-Sant Andreu Comtal | 156,90 | 0 | 0 | INCIDENT | AT DANGER SIGNAL PASSED/ NEAR MISS | TRAIN | STATION | CLOSED | 3 |
| ADIF | COMSA RAIL TRANSPORT | 0041/08 | 10/09/2008 | Villarreal | Castellón | 600 Valencia San Vicente de Calders | 62,095 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | STATION | CLOSED | 1 |
| ADIF | RENFE | 0042/08 | 12/09/2008 | Martorell | Barcelona | 240 San Vicente de Calders-L´Hospitalet de Llobregat | 73,130 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | STATION | CLOSED | 0 |
| ADIF | RENFE | 0043/08 | 25/09/2008 | Bobadilla | Málaga 4 | 26 Granada-Fuente Piedra | 14,250 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 1 |
| FEVE | FEVE | 0044/08 | 25/09/2008 | Balmaseda | Vizcaya | 1031 León-Bilbao | 284,050 | 0 | 0 | ACCIDENT | COLLISION | TRAIN | STATION | CLOSED | 1 |
| ADIF | RENFE | 0045/08 | 28/09/2008 | San Fernando | Cádiz | 400 Alcázar de San Juan-Cádiz | 140,750 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 0 |
| FEVE | FEVE | 0046/08 | 02/10/2008 | Cistierna | León | 1031 León-Bilbao | 53,725 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 2 |
| FEVE | FEVE | 0048/08 | 12/10/2008 | Pravia | Asturias | 1011 Ferrol-Oviedo | 0,287 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | STATION | CLOSED | 3 |
| ADIF | RENFE | 0049/08 | 13/10/2008 | La Encina | Alicante | 300 Madrid-Valencia | 375,740 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | STATION | CLOSED | 1 |
| ADIF | RENFE | 0050/08 | 15/10/2008 | El Caleyo | Asturias | 130 Venta de Baños-Gijón | 134,396 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | STATION | CLOSED | 1 |
| ADIF | RENFE | 0051/08 | 18/10/2008 | Figueres | Gerona | 270 Portbou –Cerbere-Barcelona-Sant Andreu Comtal | 247,215 | 1 | 0 | ACCIDENT | LEVEL CROSSING | PERSON | STATION | CLOSED | 0 |
| ADIF | RENFE | 0052/08 | 20/10/2008 | Santa María de Huerta | Soria | 200 Madrid-Barcelona | 191,876 | 1 | 0 | ACCIDENT | LEVEL CROSSING | PERSON | STATION | CLOSED | 1 |
| ADIF | COMSA RAIL TRANSPORT | 0053/08 | 23/10/2008 | Villalba de Guadarrama | Madrid | 100 Madrid-Hendaya | 38,900 | 0 | 0 | INCIDENT | AT DANGER SIGNAL PASSED / NEAR MISS | TRAIN | STATION | CLOSED | 4 |
| ADIF | RENFE | 0054/08 | 24/10/2008 | Moncófar | Castellón | 600 Valencia-San Vicente de Calders | 47,407 | 0 | 0 | ACCIDENT | DERAILMENT | TRAIN | STATION | CLOSED | 5 |
| ADIF | RENFE | 0056/08 | 02/11/2008 | Hostalric | Barcelona | 270 Portbou-Cerbere –Barcelona-Sant Andreu Comtal | 170,925 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | HALT | CLOSED | 1 |
| ADIF | RENFE | 0057/08 | 03/11/2008 | Cardedeu | Barcelona | 270 Portbou-Cerbere –Barcelona-Sant Andreu Comtal | 143,105 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | STATION | CLOSED | 1 |
| ADIF | RENFE | 0059/08 | 08/11/2008 | Tudela de Navarra | Navarra | 700 Casetas-Intermodal Abando Indalecio Prieto | 77,050 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | STATION | CLOSED | 0 |
| ADIF | RENFE | 0060/08 | 09/11/2008 | Daimiel (El Campillo) | Ciudad Real | 522 Manzanares-Ciudad Real | 226,669 | 1 | 0 | ACCIDENT | LEVEL CROSSING | MOTOR VEHICLE | LEVEL CROSSING | CLOSED | 0 |
| ADIF | RENFE | 0061/08 | 10/11/2008 | Monforte de Lemos | Lugo | 810 Vigo-Monforte de Lemos | 2,455 | 1 | 0 | ACCIDENT | LEVEL CROSSING | PERSON | LEVEL CROSSING | CLOSED | 1 |
| FEVE | FEVE | 0062/08 | 10/11/2008 | El Entrego | Asturias | 1061 Gijón-Laviana | 41,094 | 0 | 0 | INCIDENT | AT DANGER SIGNAL PASSED / NEAR MISS | TRAIN | TRACK | CLOSED | 0 |
| ADIF | RENFE | 0063/08 | 22/11/2008 | La Flecha | Salamanca | 122 Ávila-Salamanca | 104,857 | 1 | 0 | ACCIDENT | PERSON STRUCK | MOTORCYCLE | TRACK | CLOSED | 0 |
| ADIF | RENFE | 0064/08 | 22/11/2008 | Alcalá de Henares | Madrid | 200 Madrid-Barcelona | 32,800 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | STATION | CLOSED | 1 |
| ADIF | RENFE | 0065/08 | 26/11/2008 | Ribaforada | Navarra | 700 Casetas-Intermodal Abando Indalecio Prieto | 70,200 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 0 |
| ADIF | CONTINENTAL RAIL | 0066/08 | 29/11/2008 | Tembleque | Toledo | 300 Madrid-Valencia | 101,270 | 1 | 0 | ACCIDENT | PERSON STRUCK | STAFF | STATION | CLOSED | 1 |
| ADIF | RENFE | 0070/08 | 14/10/2008 | Leganés | Madrid | 500 Madrid-Valencia de Alcántara | 13,171 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | STATION | CLOSED | 0 |
| ADIF | RENFE | 0071/08 | 13/12/2008 | Calella | Barcelona | 276 Maçanet-Massanes-Barcelona -Sagrera | 48,524 | 1 | 0 | ACCIDENT | LEVEL CROSSING | PERSON | STATION | CLOSED | 1 |
| ADIF | RENFE | 0072/08 | 13/12/2008 | Vitoria | Álava | 100 Madrid-Hendaya | 494,315 | 1 | 0 | ACCIDENT | PERSON STRUCK | PERSON | TRACK | CLOSED | 0 |
| ADIF | RENFE | 0074/08 | 20/12/2008 | Salou | Tarragona | 600 Valencia-San Vicente de Calders | 263,208 | 1 | 0 | ACCIDENT | LEVEL CROSSING | PERSON | STATION | CLOSED | 0 |

4.  INVESTIGATION OF ACCIDENTS AND INCIDENTS

## Events investigated on the General Interest Rail Network

The following Table shows the monthly distribution of the accidents and incidents investigated during the year, grouped by the rail infrastructure managers responsible for same.

| Month | ADIF | FEVE | Total accidents - incidents |
| --- | --- | --- | --- |
| January | 3 |  | 3 |
| February | 2 |  | 2 |
| March | 2 |  | 2 |
| April | 2 |  | 2 |
| May | 9 |  | 9 |
| June | 5 |  | 5 |
| July | 5 |  | 5 |
| August | 1 |  | 1 |
| September | 5 | 1 | 6 |
| October | 7 | 2 | 9 |
| November | 9 | 1 | 10 |
| December | 3 |  | 3 |
| Total | 53 | 4 | 57 |

The highest level of accidents investigated was in November, and the lowest was in August, the monthly average being 4.7.

January     February     March       April        May	June         July       August      September  October    November      December

ADIF	 FEVE

In the total number of accidents and incidents investigated there were 53 casualties, of which 52 were fatal. The monthly breakdown, by network and by category of event, is given in the Table below and in the following diagram:

|  | Serious accident |  |  |  | Accident |  |  |  | Incident |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ADIF |  | FEVE |  | ADIF |  | FEVE |  | ADIF |  | FEVE |  |  |  |
| Month | Fatality | Serious injury | Fatality | Serious injury | Fatality | Serious injury | Fatality | Serious injury | Fatality | Serious injury | Fatality | Serious injury | Total fatalities | Total serious injuries |
| January |  |  |  |  | 3 |  |  |  |  |  |  |  | 3 |  |
| February | 1 |  |  |  | 1 |  |  |  |  |  |  |  | 2 |  |
| March |  |  |  |  | 2 |  |  |  |  |  |  |  | 2 |  |
| April |  |  |  |  | 2 |  |  |  |  |  |  |  | 2 |  |
| May |  |  |  |  | 1 |  |  |  |  |  |  |  | 1 |  |
| June |  |  |  |  | 5 | 1 |  |  |  |  |  |  | 5 | 1 |
| July |  |  |  |  | 3 |  |  |  |  |  |  |  | 3 |  |
| August |  |  |  |  | 1 |  |  |  |  |  |  |  | 1 |  |
| September | 1 |  |  |  | 4 |  |  |  |  |  |  |  | 5 |  |
| October |  |  |  |  | 5 |  | 2 |  |  |  |  |  | 7 |  |
| November |  |  |  |  | 9 |  |  |  |  |  |  |  | 9 |  |
| December |  |  |  |  | 3 |  |  |  |  |  |  |  | 3 |  |
| Total | 2 | 0 | 0 | 0 | 48 | 1 | 2 | 0 | 0 | 0 | 0 | 0 | 52 | 1 |

The average number of fatalities in the events investigated was 4 per month, with just one serious injury in 12 months, the latter figure being insufficient to establish any general statistical values.

January      February     March        April         May           June        July          August      September   October    November   December

Adif fatality		Feve fatality		   Adif serious injury	           Feve serious injury

4.  INVESTIGATION OF ACCIDENTS AND INCIDENTS

## Analysis by category

### 1.	Serious accidents

#### 1.1	Accidents due to collision

Two serious accidents occurred and were investigated in the course of 2008 on the rail network managed by Adif.

These two serious accidents were the result of trains colliding with a road vehicle and with a piece of track machinery (backhoe excavator). Both collisions resulted in deaths.

In one of the collisions the presence of the road vehicle on the track occurred purely by chance when the vehicle ran on to the track after being involved in a road accident.

In the other collision the presence of the track machinery was due to infrastructure maintenance work being carried out at the time of the accident.

The map on the following page shows the location on the network and the rail line where the two serious accidents which occurred in 2008 took place.

|  |  | Category | Line | K.P. | Location on map |
| --- | --- | --- | --- | --- | --- |
| SERIOUS ACCIDENT | Adif | Collision | 100 Madrid-Hendaya | 279,000 | 1 |
|  |  |  | 700 Casetas-Intermodal Abando Indalecio Prieto | 218,654 | 2 |

| Red ferroviaria de ancho U.I.C | U.I.C. gauge rail network |  | Vías verdes | Green ways |
| --- | --- | --- | --- | --- |
| Línea de Alta Velocidad | High-speed line |  | Vía verde acondicionada | Prepared green way |
| Red ferroviaria ancho Ibérico | Iberian gauge rail network |  | Vía verde potencial sin carril | Potential green way without track |
| Vía Doble | Dual track |  | Límites administrativos | Administrative boundaries |
| Vía Única | Single track |  | Límite Nacional | National boundary |
| Vía Electrificada | Electrified track |  | Límite Autónomo | Boundary of Autonomous Region |
| Otras redes ferroviarias | Other rail networks |  |  |  |
| F.E.V.E. | F.E.V.E. |  |  |  |
| Ferrocarriles Autónomos | Autonomous railways |  |  |  |

Cartographic base: Adif Communications and External Relations Directorate

4.  INVESTIGATION OF ACCIDENTS AND INCIDENTS

### 2.	Accidents

#### 2.1	Persons struck by rolling stock

During 2008 there were 35 accidents investigated involving persons being struck by rolling stock, 33 of which took place on the network administered by Adif and 2 on the network administered by Feve. Within this category,  17 occurred on open track and 18 at stations or halts, a third of these (6) while crossing between platforms.

|  | Person struck |  |  |
| --- | --- | --- | --- |
|  | Station - halt | Open track | Total |
| Adif | 17 | 16 | 33 |
| Feve | 1 | 1 | 2 |
| Total | 18 | 17 | 35 |

These events resulted in 35 deaths, of which 25 were male (71.43%)

|  | Adif |  | Feve |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Male | Female | Male | Female | Total by age | % |
| < 25 years | 3 | 2 |  |  | 5 | 14.29 |
| 25-50 years | 11 | 3 | 1 |  | 15 | 42.86 |
| 50-70 years | 6 | 3 |  |  | 9 | 25.71 |
| > 70 years | 4 | 1 |  | 1 | 6 | 17.14 |
|  | 24 | 9 | 1 | 1 | 35 | 100 |

This category excludes persons struck on level crossings (which are included under “Level crossing accidents”) and suicides, none of which were the subject of an investigation.

In all cases except one, the cause of the accident was the inappropriate behaviour of the victims.

The map on the following page shows the location on the network and the rail line where the 35 accidents where persons were struck by rolling stock during 2008 took place.

|  |  |  |  |  | Person struck |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Category | Line | K.P. | Station - halt | Open track | Location on map |
| Accident | Adif | Person struck by rolling stock | 100 Madrid-Hendaya | 20,100 |  | 1 | 1 |
|  |  |  |  | 494,315 |  | 1 | 2 |
|  |  |  |  | 594,378 |  | 1 | 3 |
|  |  |  |  | 608,155 |  |  | 4 |
|  |  |  |  | 615,049 |  |  | 5 |
|  |  |  | 122 Ávila-Salamanca | 104,857 |  | 1 | 6 |
|  |  |  | 130 Venta de Baños-Gijón | 134,396 |  |  | 7 |
|  |  |  | 200 Madrid-Barcelona | 149,950 |  | 1 | 8 |
|  |  |  |  | 624,800 | 1 |  | 9 |
|  |  |  |  | 32,800 | 1 |  | 10 |
|  |  |  | 210 Miraflores-Tarragona | 470,754 | 1 |  | 11 |
|  |  |  | 240 San Vicente de Calders-L’Hospitalet de Llobregat | 73,130 | 1 |  | 12 |
|  |  |  | 270 Portbou-CerbereBarcelona-San Andreu Comtal | 143,105 | 1 |  | 13 |
|  |  |  |  | 170,925 | 1 |  | 14 |
|  |  |  | 276 Maçanet-Massanes-Barcelona-Sagrera | 26,100 | 1 |  | 15 |
|  |  |  |  | 3,286 |  | 1 | 16 |
|  |  |  | 300 Madrid-Valencia | 202,400 |  | 1 | 17 |
|  |  |  |  | 101,270 | 1 |  | 18 |
|  |  |  |  | 13,100 |  | 1 | 19 |
|  |  |  |  | 375,740 | 1 |  | 20 |
|  |  |  |  | 109,900 |  | 1 | 21 |
|  |  |  | 330 La Encina-Alicante | 396,675 |  | 1 | 22 |
|  |  |  | 400 Alcázar de San Juan-Cádiz | 140,750 |  | 1 | 23 |
|  |  |  | 426 Granada- Fuente Piedra | 14,250 |  | 1 | 24 |
|  |  |  | 436 Málaga- Fuengirola | 8,870 |  | 1 | 25 |
|  |  |  | 440 Majarabique Estación- Huelva Término | 24,900 |  | 1 | 26 |
|  |  |  | 500 Madrid-Valencia de Alcántara | 13,171 | 1 |  | 27 |
|  |  |  | 600 Valencia- San Vicente de Calders | 62,095 | 1 |  | 28 |
|  |  |  | 700 Casetas-Intermodal Abando Indalecio Prieto | 146,950 | 1 |  | 29 |
|  |  |  |  | 70,200 |  | 1 | 30 |
|  |  |  |  | 77,050 | 1 |  | 31 |
|  |  |  | 720 Intermodal Abando Indalecio Prieto- | 5,348 | 1 |  | 32 |
|  |  |  | 800 La Coruña-León | 310,412 |  | 1 | 33 |
|  |  |  | 1011 Ferrol-Oviedo | 0,287 | 1 |  | 34 |
|  | Feve |  | 1031 León-Bilbao | 53,725 |  | 1 | 35 |

STATION - HALT

OPEN TRACK		   OPEN TRACK

| Red ferroviaria de ancho U.I.C | U.I.C. gauge rail network |  | Vías verdes | Green ways |
| --- | --- | --- | --- | --- |
| Línea de Alta Velocidad | High-speed line |  | Vía verde acondicionada | Prepared green way |
| Red ferroviaria ancho Ibérico | Iberian gauge rail network |  | Vía verde potencial sin carril | Potential green way without track |
| Vía Doble | Dual track |  | Límites administrativos | Administrative boundaries |
| Vía Única | Single track |  | Límite Nacional | National boundary |
| Vía Electrificada | Electrified track |  | Límite Autónomo | Boundary of Autonomous Region |
| Otras redes ferroviarias | Other rail networks |  |  |  |
| F.E.V.E. | F.E.V.E. |  |  |  |
| Ferrocarriles Autónomos | Autonomous railways |  |  |  |

Cartographic base: Adif Communications and External Relations Directorate

4.  INVESTIGATION OF ACCIDENTS AND INCIDENTS

#### 2.2	Collisions

One accident due to a collision was investigated in 2008 on the rail network managed by Feve.

A car, listed as a historic vehicle, struck a set of buffers at Balmaseda goods station after its brakes failed. The cause of the accident was the loss of the braking system on the car, causing it to swerve. There were no fatalities.

#### 2.3	Derailments

Two accidents due to derailment were investigated in 2008 on the network administered by Adif.

One of these occurred when a passenger train was partially derailed (the first seven carriages) at Medina del Campo station. The cause was inappropriate speed on this section of line.

The second accident occurred when a goods train was derailed when leaving Moncófar station, the locomotive and six wagons being derailed. The accident was caused by breakage of the left wheel on the third axle.

The map on the following page shows the location on the network and the rail line where the single accident due to collision and the two derailments investigated in 2008 took place.

|  |  | Category | Line | K.P. | Location on map |
| --- | --- | --- | --- | --- | --- |
| ACCIDENT | Adif | Derailment | 100 Madrid-Hendaya | 208,312 | 1 |
|  |  |  | 600 Valencia-San Vicente de Calders | 47,407 | 2 |
|  | Feve | Collision | 1031 León-Bilbao | 284,05 | 3 |

DERAILMENT

COLLISION

| Red ferroviaria de ancho U.I.C | U.I.C. gauge rail network |  | Vías verdes | Green ways |
| --- | --- | --- | --- | --- |
| Línea de Alta Velocidad | High-speed line |  | Vía verde acondicionada | Prepared green way |
| Red ferroviaria ancho Ibérico | Iberian gauge rail network |  | Vía verde potencial sin carril | Potential green way without track |
| Vía Doble | Dual track |  | Límites administrativos | Administrative boundaries |
| Vía Única | Single track |  | Límite Nacional | National boundary |
| Vía Electrificada | Electrified track |  | Límite Autónomo | Boundary of Autonomous Region |
| Otras redes ferroviarias | Other rail networks |  |  |  |
| F.E.V.E. | F.E.V.E. |  |  |  |
| Ferrocarriles Autónomos | Autonomous railways |  |  |  |

Cartographic base: Adif Communications and External Relations Directorate

4.  INVESTIGATION OF ACCIDENTS AND INCIDENTS

#### 2.4.	Level crossing accidents

The Ministerial Order of 2 August 2001, regulating the removal and protection of level crossings, classifies level crossings into seven types.

| CLASSIFICATION OF LEVEL CROSSINGS ( Ministerial Order of 2 August 2001) |  |
| --- | --- |
| Type | Characteristics |
| Category A | Protected only by fixed signage |
| Category B | Protected by luminous and acoustic signage |
| Category C | Protected with automatic or interlocking half barriers, double half barriers or barriers |
| Category D | Locally monitored by a railway employee |
| Category E | Manually controlled by crossing keeper |
| Category F | Pedestrians or pedestrians and cattle only |
| Category P | Private |

The inventory of level crossings in the general interest rail network, as at 31 December 2008, is set out in the following Table, by type:

|  | Adif | Feve | Total |
| --- | --- | --- | --- |
| Category A | 1.205 | 590 | 1.795 |
| Category B | 484 | 58 | 542 |
| Category C | 533 | 176 | 709 |
| Category D | 17 |  | 17 |
| Category E | 2 | 1 | 3 |
| Category F | 74 | 69 | 143 |
| Category P | 384 | 130 | 514 |
|  | 2.699 | 1.024 | 3.723 |

Of the 3 723 crossings inventoried,  72% are administered by Adif and the remaining 28% are on the network administered by Feve. Of that total:

48% (Category A) are protected only by fixed signage;

38% (Categories B, C, D, E and F) have some form of additional protection in addition to the fixed signage, ranging from luminous and acoustic signalling to barriers, chains, etc.;

14% (Category P) are private and are regulated by the terms of their individual permits.

There were 14 level crossing accidents investigated in 2008, all of them on the network administered by Adif. In 8 of these cases (57%) the casualties were pedestrians.

|  | Adif |  |
| --- | --- | --- |
|  | Vehicle | Pedestrian |
| Category A | 2 |  |
| Category B | 1 |  |
| Category C | 2 | 7 |
| Category D |  |  |
| Category E |  |  |
| Category F |  | 1 |
| Category P | 1 |  |
|  | 6 | 8 |

These accidents caused 15 fatalities. Most occurred on crossings with Type C protection. In the following two Tables the casualties are grouped by age bracket and by gender:

|  | Adif |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Category A | Category B | Category C | Category F | Category P |
| < 25 years |  |  | 2 |  |  |
| 25-50 years | 1 | 1 | 1 | 1 | 1 |
| 50-70 years | 1 |  | 1 |  |  |
| > 70 years | 1 |  | 5 |  |  |
|  | 3 | 1 | 9 | 1 | 1 |

The age bracket with the highest accident rate is the one for persons over 70, followed by the 25-50 year bracket. As regards classification by gender, 80% of the casualties were male.

|  | Adif |  |  |  |
| --- | --- | --- | --- | --- |
|  | Male | Female | Total by age | % |
| < 25 years | 1 | 1 | 2 | 13.34 |
| 25-50 years | 4 | 1 | 5 | 33.32 |
| 50-70 years | 2 |  | 2 | 13.34 |
| > 70 years | 5 | 1 | 6 | 40.00 |
|  | 12 | 3 | 15 | 100 |

4.  INVESTIGATION OF ACCIDENTS AND INCIDENTS

The map on the following page shows the locations on the network and the rail line where the level crossing accidents investigated in 2008 occurred.

|  |  | Category | Line | K.P. | Location on map |
| --- | --- | --- | --- | --- | --- |
| ACCIDENT | Adif | Level crossing | 200 Madrid-Barcelona | 214,200 | 1 |
|  |  |  |  | 191,876 | 2 |
|  |  |  | 240 San Vicente de Calders-L´Hospitalet de Llobregat | 88,906 | 3 |
|  |  |  | 270 Portbou-Cerbere-Barcelona-Sant Andreu Comtal | 148,000 | 4 |
|  |  |  |  | 247,215 | 5 |
|  |  |  |  | 247,215 | 6 |
|  |  |  | 276 Maçanet-Massanes-Barcelona-Sagrera | 48,524 | 7 |
|  |  |  | 310 Aranjuez-Valencia | 39,950 | 8 |
|  |  |  | 400 Alcázar de San Juan-Cádiz | 429,825 | 9 |
|  |  |  |  | 511,635 | 10 |
|  |  |  | 522 Manzanares-Ciudad Real | 226,669 | 11 |
|  |  |  | 600 Valencia-San Vicente de Calders | 263,208 | 12 |
|  |  |  | 800 La Coruña-León | 263,566 | 13 |
|  |  |  | 810 Vigo-Monforte de Lemos | 2,455 | 14 |

| Red ferroviaria de ancho U.I.C | U.I.C. gauge rail network |  | Vías verdes | Green ways |
| --- | --- | --- | --- | --- |
| Línea de Alta Velocidad | High-speed line |  | Vía verde acondicionada | Prepared green way |
| Red ferroviaria ancho Ibérico | Iberian gauge rail network |  | Vía verde potencial sin carril | Potential green way without track |
| Vía Doble | Dual track |  | Límites administrativos | Administrative boundaries |
| Vía Única | Single track |  | Límite Nacional | National boundary |
| Vía Electrificada | Electrified track |  | Límite Autónomo | Boundary of Autonomous Region |
| Otras redes ferroviarias | Other rail networks |  |  |  |
| F.E.V.E. | F.E.V.E. |  |  |  |
| Ferrocarriles Autónomos | Autonomous railways |  |  |  |

Cartographic base: Adif Communications and External Relations Directorate

4.  INVESTIGATION OF ACCIDENTS AND INCIDENTS

#### 2.5	Accidents due to fire

No accidents due to fire were investigated in 2008

#### 2.6	Other accidents

No other type of accident was investigated in 2008

### 3.	Incidents

As already mentioned in the Introduction, during its first year of activity the Rail Accident Investigation Commission undertook the investigation of those incidents which, by their nature and potential risk, had repercussions for rail safety. Prior to the entry into force of the Reglamento sobre seguridad en la circulación de la RFIG [Regulations on Traffic Safety in the General Interest Rail Network] such events were not investigated by the competent investigation body.

Three incidents were investigated during 2008: three at danger signals passed resulting in a near collision.

Two of these incidents occurred on the network administered by Adif:

- The first incident occurred at Sant Celoni station when a local train went through the distant signal which was giving an advance stop warning and encountered a goods train stopped at the home signal, which was indicating stop. This was the result of a failure to comply with the Standing Orders governing the work on the replacement of the interlocking system which was in progress at the time.

- The second incident occurred when a light engine passed the home signal at Villalba de Guadarrama station (which was giving a Stop indication). The cause was insufficient braking, which failed to reduce the speed adequately before the stop signal.

The third incident occurred on the network managed by Feve:

- A local train passed the stop signal at El Entrego station, resulting in a near collision. The cause was failure to comply with the Reglamento de Circulación de Trenes [Train Movement Regulations] and Reglamento de Señales [Signalling Regulations] issued by Feve.

The map on the following page shows the locations on the network and the rail line where the incidents investigated in 2008 occurred.

|  |  | Category | Line | K.P. | Location on map |
| --- | --- | --- | --- | --- | --- |
| ACCIDENT | Adif | At danger signal passed / near miss | 100 Madrid-Hendaya | 38,900 | 1 |
|  |  | At danger signal passed / near miss | 270 Portbou-Cerbere-Barcelona-Sant Andreu Comtal | 156,90 | 2 |
|  | Feve | At danger signal passed / near miss | 1031 León-Bilbao | 41,094 | 3 |

| Red ferroviaria de ancho U.I.C | U.I.C. gauge rail network |  | Vías verdes | Green ways |
| --- | --- | --- | --- | --- |
| Línea de Alta Velocidad | High-speed line |  | Vía verde acondicionada | Prepared green way |
| Red ferroviaria ancho Ibérico | Iberian gauge rail network |  | Vía verde potencial sin carril | Potential green way without track |
| Vía Doble | Dual track |  | Límites administrativos | Administrative boundaries |
| Vía Única | Single track |  | Límite Nacional | National boundary |
| Vía Electrificada | Electrified track |  | Límite Autónomo | Boundary of Autonomous Region |
| Otras redes ferroviarias | Other rail networks |  |  |  |
| F.E.V.E. | F.E.V.E. |  |  |  |
| Ferrocarriles Autónomos | Autonomous railways |  |  |  |

Cartographic base: Adif Communications and External Relations Directorate

# 5.	CONCLUSIONS

## Causes of the events investigated

The following Table shows the causes of the events investigated, grouped by category of event and type.

| Cause attributed to | Category | Network | Type | Cause | Total events | Total casualties |
| --- | --- | --- | --- | --- | --- | --- |
| RAILWAY | Accident | Adif | Person struck | Broken door | 1 | 1 |
|  |  |  | Derailment | Non-compliance with General Traffic Regulations (RGC) | 1 | 0 |
|  |  |  |  | Broken wheel | 1 | 0 |
|  |  | Feve | Collision | Brake failure | 1 | 0 |
|  |  |  |  |  | 4 | 1 |
|  | Serious accident | Adif | Collision | Non-compliance with General Traffic Regulations (RGC) | 1 | 1 |
|  |  |  |  |  | 1 | 1 |
|  | Incident | Adif | At danger signal passed / near miss | Non-compliance with regulations | 1 | 0 |
|  |  |  | At danger signal passed / near miss | Inadequate braking | 1 | 0 |
|  |  | Feve | At danger signal passed / near miss | Non-compliance with General Traffic Regulations (RGC) | 1 | 0 |
|  |  |  |  |  | 3 | 0 |
|  |  |  |  | Total | 8 | 2 |
| THIRD PARTIES | Accident | Adif | Level crossing | Fatality: very close to track | 1 | 1 |
|  |  |  |  | Fatality: did not see train | 5 | 6 |
|  |  |  |  | Imprudent behaviour: preventive measures in operation | 8 | 8 |
|  |  |  | Person struck | Fatality: did not see train | 1 | 1 |
|  |  |  |  | Intrusion into unauthorised area | 25 | 25 |
|  |  |  |  | Improper use of crossing between platforms | 3 | 3 |
|  |  |  |  | Improper use of crossing between platforms | 3 | 3 |
|  |  | Feve | Person struck | Intrusion into unauthorised area | 2 | 2 |
|  |  |  |  |  | 48 | 49 |
|  | Serious accident | Adif | Collision | Chance | 1 | 1 |
|  |  |  |  |  | 1 | 1 |
|  |  |  |  | Total | 49 | 50 |
|  |  |  |  | Combined Total | 57 | 52 |

According to the investigations carried out, 86% of events (49 out of 57) and 96% of fatalities (50 out of 52) were attributable to inappropriate behaviour of persons unconnected with the railway.

92.30% of the accidents were attributable to third parties: the main cause (56.25%) was the victim being in a prohibited location, followed by failure to comply with signs prohibiting entry (16.66%).

Of the 8 events attributable to the railway, half were caused by failure to comply with regulations and the other half were due to problems with rolling stock.

# 6.	COMPARISON

The following Tables give the details of rail accidents and incidents, casualties (deaths and serious injuries) and the recommendations made for each of the events which, since the entry into force of the Ley del Sector Ferroviario [Rail Sector Law] in December 2004, have been investigated under provisions contained in that Law and subsequently further elaborated in the Reglamento del Sector Ferroviario [Rail Sector Regulations] and the Reglamento sobre seguridad en la circulación [Traffic Safety Regulations] (September 2007).

The number of casualties (deaths and serious injuries) fell substantially in 2008, from an average of 63 in the years 2005-2007 to 53 in 2008.

Events investigated in 2005-2008

|  | 2005 |  |  | 2006 |  |  | 2007 |  |  | 2008 |  |  | Combined total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ADIF | FEVE | Total | ADIF | FEVE | Total | ADIF | FEVE | Total | ADIF | FEVE | Total |  |
| PERSON STRUCK | 34 |  | 34 | 36 | 1 | 37 | 43 | 3 | 46 | 33 | 2 | 35 | 152 |
| COLLISION |  |  |  | 2 |  | 2 |  |  |  | 2 | 1 | 3 | 5 |
| DERAILMENT |  |  |  | 1 |  | 1 |  | 1 | 1 | 2 |  | 2 | 4 |
| LEVEL CROSSING | 20 | 4 | 24 | 14 |  | 14 | 16 | 4 | 20 | 14 |  | 14 | 72 |
| AT DANGER SIGNAL PASSED / NEAR MISS |  |  |  |  |  |  |  |  |  | 2 | 1 | 3 | 3 |
|  | 54 | 4 | 58 | 53 | 1 | 54 | 59 | 8 | 67 | 53 | 4 | 57 | 236 |

Casualties in events investigated in 2005-2008

|  |  | 2005 |  |  | 2006 |  |  | 2007 |  |  | 2008 |  |  | Combined total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | Death ----------- Serious injury | ADIF | FEVE | Total | ADIF | FEVE | Total | ADIF | FEVE | Total | ADIF | FEVE | Total |  |
| PERSON STRUCK |  | 35 |  | 35 | 38 | 1 | 39 | 46 | 3 | 49 | 33 | 2 | 35 | 158 |
|  |  | 1 |  | 1 | 2 |  | 2 |  |  |  | 1 |  | 1 | 4 |
| COLLISION |  |  |  |  | 2 |  | 2 |  |  |  | 2 |  | 2 | 4 |
| DERAILMENT |  |  |  |  | 7 |  | 7 |  |  |  |  |  |  |  |
|  |  |  |  |  | 6 |  | 6 |  |  |  |  |  |  |  |
| LEVEL CROSSING |  |  |  |  |  |  | 16 | 19 | 4 | 23 | 15 |  | 15 | 81 |
|  |  |  |  |  |  |  |  | 3 |  | 3 |  |  |  | 3 |
| AT DANGER SIGNAL PASSED / NEAR MISS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total deaths |  | 58 | 4 | 62 | 63 | 1 | 64 | 65 | 7 | 72 | 50 | 2 | 52 | 250 |
| Total serious injuries |  | 1 |  | 1 | 8 |  | 8 | 3 |  | 3 | 1 |  | 1 | 13 |

Recommendations made in 2005-2008 *

|  | 2005 |  |  | 2006 |  |  | 2007 |  |  | 2008 |  |  | Combined total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TYPE | ADIF | FEVE | Total | ADIF | FEVE | Total | ADIF | FEVE | Total | ADIF | FEVE | Total |  |
| PERSON STRUCK | 36 |  | 36 | 45 | 2 | 47 | 82 | 8 | 90 | 19 | 5 | 24 | 197 |
| COLLISION |  |  |  | 2 |  | 2 |  |  |  | 1 | 1 | 2 | 4 |
| DERAILMENT |  |  |  | 4 |  | 4 |  | 8 | 8 | 5 |  | 5 | 17 |
| LEVEL CROSSING | 21 | 4 | 25 | 14 |  | 14 | 26 | 12 | 38 | 7 |  | 7 | 84 |
| AT DANGER SIGNAL PASSED / NEAR MISS |  |  |  |  |  |  |  |  |  | 8 |  | 8 | 8 |
|  | 57 | 4 | 61 | 65 | 2 | 67 | 108 | 28 | 136 | 40 | 6 | 46 | 310 |

*  	The recommendations are not listed according to the party to whom the recommendation is directed but on the basis of the manager of the network where the events occurred.

# 7.	REPORTS AND RECOMMENDATIONS ISSUED

## Recommendations contained in the accident and incident reports

As established in the regulations, the purpose of rail accident and incident investigation is to contribute to improving the management of rail safety, reducing the risk of accidents and incidents by formulating conclusions and recommendations which are then communicated to the various parties involved in the rail sector.

57 investigations of events occurring on the General Interest Rail Network were initiated and completed during 2008, and a total of 46 recommendations were made. The Rail Accident Investigation Commission also considered it appropriate to issue a recommendation on an incident which, although it was not investigated, attracted the interest of the Commission. (see Case 0055/08 in the Table on the following page).

The graph below shows the recommendations made to each party. The majority of the recommendations were addressed to the competent administrations - 15 - (32.61% of the total) followed by those addressed to the infrastructure administrator Adif - 14 -  (30.43% of the total).

Parties receiving Recommendations

| Dirección General Ferrocarriles (DGF); 4 | Railways Directorate : 4 |
| --- | --- |
| Adif; 14 | Adif : 14 |
| Feve; 2 | Feve : 2 |
| Renfe Operadora; 5 | Renfe Operadora : 5 |
| Comsa Rail Transport; 2 | Comsa Rail Transport : 2 |
| Centros de mantenimiento; 2 | Maintenance centres : 2 |
| Administradores de la Infraestructura; 1 | Infrastructure administrators : 1 |
| Empresas Ferroviarias; 1 | Rail companies : 1 |
| Ayuntamientos; 2 | Municipal Councils : 2 |
| Administración competente; 15 | Competent administration : 15 |

Below is a summary of all the investigations undertaken and recommendations issued.

7.  REPORTS AND RECOMMENDATIONS ISSUED

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0001/08 | 10.01.2008 | 400 Alcázar de San Juan-Cádiz |  | Adif | Renfe Operadora | Level crossing |
| A goods train struck a cyclist at a Category C level crossing at Alcolea de Córdoba station. The victim crossed the level crossing when the half barriers were down. The accident was caused by the victim attempting to cross the level crossing when this was prohibited and this prohibition was adequately signposted. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Railways Directorate (DGF) |  |  |  |
| 01/08-1* It is proposed that consideration be given to removing the level crossing at this location, which has a high level of traffic (8 776 vehicles per day). As the DGF Level Crossings Database (29.02.08) indicates the intention of removing this level crossing, it is recommended that this process be expedited. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0002/08 | 26.01.2008 | 800 La Coruña - León |  | Adif | Renfe Operadora | Person struck |
| A medium-haul passenger train travelling between Montefurado and A Rúa Petín stations struck a person on the track. The victim was crossing the line from left to right in a prohibited location. The accident was caused by the victim crossing the line at a point not suitable for crossing, while the train was travelling down the line. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 02/08 0* Given the nature of the accident, there are no recommendations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0003/08 | 22.02.2008 | 700 Casetas-Intermodal Abando Indalecio Prieto |  | Adif | Renfe Operadora | Collision |
| A light engine collided with a road vehicle at the Salbio halt. The vehicle plunged from the overpass of the A-625 motorway after breaking through the protective barriers. The sole occupant of the vehicle died in the accident. The accident was caused by the intrusion onto the line of the vehicle which had already suffered an accident, and there was no way the locomotive could avoid hitting it. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 03-08-0* No recommendations made. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0004/08 | 14.03.2008 | 270 Portbou-Cerbere-Barcelona-Sant Andreu Comtal |  | Adif | Renfe Operadora | Level crossing |
| A local passenger train entering Llinars del Vallés station suddenly encountered two motorcyclists crossing the Category C level crossing situated inside the station. The half barriers were closed. The first rider managed to cross before the train arrived but the second rider was struck. The accident was caused by improper use of the level crossing, swerving around the closed barriers, while the train was moving. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 04/08-0* Given the nature of the accident no specific recommendations were made for this particular accident. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0006-08 | 21.03.2008 | 276 Maçanet-Massanes-Barcelona-Sagrera |  | Adif | Renfe Operadora | Person struck |
| A local passenger train crossing the rail bridge over the river Besós (Barcelona) struck a person who was inappropriately walking on the bridge. The accident was caused by the victim straying on to the track at an unauthorised location. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Adif |  |  |  |
| 06-08-1* Elimination of the unsafe gap where the victim gained access. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0009-08 | 16.04.2008 | 240 San Vicente de Calders-L´Hospitalet de Llobregat |  | Adif | Renfe Operadora | Level crossing |
| A local passenger train entering the station at Sant Feliú de Llogbregat suddenly encountered a pedestrian on the Category C level crossing located inside the station. The half barriers were closed. The person died as a result of being struck. The cause of the accident was the victim using the level crossing when the signals indicated that crossing was prohibited. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 09-08-0* As there is an overhead pedestrian crossing and plans are in hand to remove the level crossing for vehicles, no specific recommendations are made for this accident. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0010-08 | 22.04.2008 | 720 Intermodal Abando Indalecio Prieto-Santurtzi |  | Adif | Renfe Operadora | Person struck |
| A goods train passing through Zorrotza station struck a person who was walking, close to the platform, along line 1 of the station. The accident was caused by the victim being on a part of the line where access is prohibited. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 10-08-0* Given the nature of the accident, there are no recommendations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0012-08 | 06.05.2008 | 210 Miraflores-Tarragona |  | Adif | Renfe Operadora | Person struck |
| A person travelling on a medium haul passenger train fell on to the track as the train was pulling out on line 2 at Fabara station where it had made a commercial stop. The driver, who saw the passenger fall in his rear view mirror, stopped the train before it hit the passenger. The victim ended up between the first and second coaches of the train. The accident was caused by the victim alighting from the train by the second door of the first carriage after the train started moving. An underlying cause was the faulty operation of the door from which the victim is presumed to have fallen. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Renfe Operadora |  |  |  |
| 12-08-1* It is recommended that Renfe Operadora, after conducting a suitable study as part of mid-term reviews to be selected by Renfe Operadora, should revise its existing maintenance plan for this type of equipment (which already includes checks on the proper operation of doors, hinges and steps) to include checks on wear and slack in the operating mechanisms (pistons, actuator rods, etc.), replacing them as necessary. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0013/08 | 09.05.2008 | 200 Madrid-Barcelona |  | Adif | Renfe Operadora | Level crossing |
| A goods train struck a vehicle with two occupants at the Category A level crossing at K.P. 214.220 on Line 200 Madrid-Barcelona. The accident was caused by the vehicle crossing the level crossing as the train was approaching, without the occupants being aware of the train’s presence. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Cetina (Zaragoza) Municipal Council |  |  |  |
| 13/08-1* As the traffic moment at this level crossing is within approved limits, it would not be appropriate to recommend its removal, but it is recommended that the municipal council be made aware of the need to provide the crossing with appropriate road signage (in accordance with the 6th additional provision of the Order of 2 August 2001 for the detailed implementation of Article 235 of the Regulations to the Ley de Ordenación de Transportes Terrestres [Law on the Management of Land Transport] in relation to the removal and protection of level crossings) |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0014/08 | 13.05.2008 | 100 Madrid-Hendaya |  | Adif | Renfe Operadora | Person struck |
| A local train struck a person at K.P. 594.380, located between the Alegia halt and Tolosa station, with fatal results. The person struck was in charge of Health and Safety for the work being done on electricity posts at the time of the accident. This work was scheduled in the weekly works notice No. 123 of the Miranda de Ebro Operations Management. The accident was caused by the person struck penetrating the clearance envelope of the track as the local train was passing. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Competent body |  |  |  |
| 14-08-1* As the accident under investigation did not result from a breach of the General Traffic Regulations, the Commission takes the view that some appropriate person should analyze whether the rules applicable to the safety of work in the vicinity of the track, and the level of monitoring being applied are adequate. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0015/08 | 13.05.2008 | 100 Madrid-Hendaya |  | Adif | Renfe Operadora | Person struck |
| At the Hernani Centro halt a long haul passenger train travelling on line I struck a person who was crossing via the boarded crossing from the platform of line II to line I. The accident was caused by the victim using the boarded crossing without noticing that a train was approaching on line I. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Competent body |  |  |  |
| 15-08-1* Promote public awareness campaigns on the responsible use of railway installations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0016/08 | 14.05.2008 | 800 La Coruña-León |  | Adif | Renfe Operadora | Level crossing |
| A levelling, aligning and tamping track machine stopped after striking a person on the Category F level crossing located at the Villadepalos halt. The accident was caused by the person struck being on the track, trying to get one of his two dogs off the track, while the machine was in motion. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 16/08-0* Given the nature of the accident and the fact that the process of removing this level crossing is near to completion, no specific recommendations are made for this particular accident. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0017/08 | 24.05.2008 | 330 La Encina-Alicante |  | Adif | Renfe Operadora | Person struck |
| Between the distant signal and the home signal at Villena station a long haul passenger train struck a person walking their dog close to the line. The accident was caused by the victim walking in a prohibited area. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 2 |  |  | 1. Adif 2. Competent body |  |  |  |
| 17/08-1* Strengthen the fencing 17/08-2* Promote public education and awareness campaigns about prohibitions and the dangers of unauthorised entry to railway installations |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0018-08 | 27.05.2008 | 310 Aranjuez-Valencia |  | Adif | Renfe Operadora | Level crossing |
| A medium haul train struck a backhoe excavator which was crossing the Category B level crossing between Santa Cruz de la Zarza and Ocaña stations. The accident was caused by the excavator crossing the level crossing while the medium haul train was in transit. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Competent body |  |  |  |
| 18/08-1* The recommendation for this accident forms part of a general recommendation on educational campaigns relating to level crossing signals. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0019/08 | 24.05.2008 | 440 Majarabique Estación-Huelva Término |  | Adif | Renfe Operadora | Person struck |
| Before arrival at Aznalcázar Pilas station a medium haul passenger train struck a person walking close to the line. The accident was caused by the victim entering a prohibited area. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1. |  |  | Competent body |  |  |  |
| 19/09-1* In this case it is appropriate to make a general recommendation concerning events of this kind: promote public education and awareness campaigns about the importance of not entering railway installations where access is not permitted. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0020/08 | 03.06.2008 | 270 Portbou-Cerbere-Barcelona-Sant Andreu Costal |  | Adif | Renfe Operadora | Level crossing |
| A medium haul passenger train struck a person on the Category C level crossing at Figueres station. The accident was caused by the victim being close to the track centre when the train crossed the level crossing. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 20/08-0* Since it is planned to remove this level crossing it is not considered necessary to make any recommendation. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0021/08 | 03.06.2008 | 400 Alcázar de San Juan - Cádiz |  | Adif | Renfe Operadora | Level crossing |
| A goods train struck a car at the Category P level crossing located between El Priorato and Lora del Río stations. The accident was caused by the vehicle crossing the level crossing while the train was in transit. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Railways Directorate (DGF) |  |  |  |
| 21/08-1* It is recommended that this level crossing be removed because it does not comply with Article 8(5) of Law 39/2003 of 17 November on the Railway Sector: ‘closure of level crossings established on private roads when the owners of these crossings do not pay proper attention to their maintenance, protection and signage’. However, in accordance with Circular Resolution 1/2008 of the Railways Directorate, relating to the plan for reviewing the authorisations for private level crossings on the General Interest Rail Network administered by Adif, it should be included in the authorisation review plan which the Infrastructure Administrator has to submit to the Railways Directorate by 31 December 2009. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0022/08 | 06.01.2008 | 100 Madrid-Hendaya |  | Adif | Renfe Operadora | Person struck |
| The driver of a local passenger train running between the stations of Pitis and Pinar de las Rozas reported to the Madrid Chamartin Control Centre that as he passed through the tunnel at K.P. 20.100 on line 100 Madrid-Hendaya he noted an impact and a metallic sound. Following instructions from the Control Centre another train, running at sight, confirmed the presence of human remains at that kilometre point. The accident was caused by the victim crossing the line at an unauthorised location. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 22/08-0* Given the nature of the accident there are no recommendations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0023/08 | 05.02.08 | 300 Madrid-Valencia |  | Adif | Renfe Operadora | Person struck |
| A passenger train travelling between San Cristóbal Industrial and Getafe Industrial stations struck a person walking along the track in the same direction as the train. The accident was caused by the victim walking in a prohibited area as the train was passing along the line. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Adif |  |  |  |
| 23/08-1* In view of the considerable number of persons being struck in the southern zone of Madrid it is recommended that a study be carried out to see whether there are any dangerous access points in the area which, if they do exist, should be eliminated. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0024/08 | 22.06.2008 | 200 Madrid-Barcelona |  | Adif | Renfe Operadora | Person struck |
| A long haul passenger train struck a person on the line. The point where the accident took place is in the immediate vicinity of a well-known leisure centre. The accident was caused by the victim being within the track clearance envelope when the train arrived. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 24/08-0* Given the nature of the accident there are no recommendations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0026/08 | 24.06.2008 | 276 Maçanet-Massanes-Barcelona-Sagrera |  | Adif | Renfe Operadora | Person struck |
| A short haul passenger train struck two persons who were walking along the track at K.P. 26.100 (at the start of the viaduct over the Riera de Argentona).There is a commercial centre in the immediate vicinity and defects have been found in the fencing (dangerous openings) , making it easy for pedestrians to gain access to the track. Conclusion: The accident was caused by the presence of the victims as the short haul train was travelling along the line. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 2 |  |  | 1. Adif 2. Municipal Councils of Cabrera de Mar and Mataró (Barcelona) |  |  |  |
| 26/08-1* Repair the breaks in the fencing which allow persons to pass from the mountain side to the seaward side on the section of track between Villasar and Mataró, and particularly in the vicinity of the bridge over the Riera de Argentona. 26/08-2* A problem of urban mobility between the municipalities of Cabrera de Mar and Mataró has been noted (the two being separated by the Riera de Argentona). It is recommended that these organisations examine the possibility of erecting a pedestrian footbridge over the Riera, to prevent pedestrians crossing via the railway bridge. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0028/08 | 30.06.2008 | 700 Casetas-Intermodal Abando Indalecio Prieto |  | Adif | Renfe Operadora | Person struck |
| A goods train struck a person at K.P. 146.950 on line 700 Casetas-Intermodal Abando Indalecio Prieto. The accident occurred at the exit from Miranda de Ebro station at switch point No. 10 on the branch towards Bilbao. Conclusion: The accident was caused by the presence of the victim in a prohibited area. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 28/08-0* Given the nature of the accident there are no recommendations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0029/08 | 05.07.2008 | 100 Madrid- Hendaya |  | Adif | Renfe Operadora | Derailment |
| The first seven carriages of a long haul passenger train were derailed, the locomotive and the remaining 16 carriages of the train remaining on the track, The train had just passed the home signal at Medina del Campo station. As a result of the derailment 9 persons were slightly injured (8 passengers and a waiter on board the train). The derailment was caused by the failure of the driver to negotiate a curved section of track in accordance with the existing signs indicating a speed restriction of 120 km/h (before the curve) and 90 km/h (entering the curve), these signs being passed at 154 km/h and 151 km/h respectively. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 29/08-0* No recommendations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0030/08 | 16.07.2008 | 300 Madrid-Valencia |  | Adif | Renfe Operadora | Person struck |
| A long haul passenger train struck a person at K.P. 108.898 on line 300 Madrid Chamartín - Valencia Estación de Nord. The accident occurred between the distant and home signals for Valencia Nord station. The accident was caused by the victim being in a prohibited area as the train passed that point. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 30/08-0* Given the nature of the accident there are no recommendations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0031/08 | 30.07.2008 | 100 Madrid- Hendaya |  | Adif | Renfe Operadora | Person struck |
| A goods train struck a person at the Andoaín Centro halt as he was crossing from the platform for line II to the platform for line I, via a boarded crossing between the platforms. According to a statement by the ticket clerk the victim had a mental disability, was a regular user of the halt and always wore headphones. The accident was caused by the victim crossing via the boarded crossing as the train was passing. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 31/08-0* In view of the proposed actions, no recommendations are made. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0032/08 | 30.07.2008 | 436 Málaga-Fuengirola |  | Adif | Renfe Operadora | Person struck |
| A local passenger train struck a person who was walking alongside another person along the track at K.P. 8.870 on line 436 Fuengirola-Málaga. The accident was caused by the victim being in a prohibited area as the passenger train was travelling along the line. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 32/08-0* Given the nature of the accident there are no recommendations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0037/08 | 29.08.2008 | 200 Madrid-Barcelona |  | Adif | Renfe Operadora | Person struck |
| A medium haul passenger train struck a person on the boarded crossing at the Segur de Calafell halt. The victim was a person of reduced mobility who was walking with a crutch. The accident was caused by the victim using the boarded crossing without being aware of the approaching train. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Competent body |  |  |  |
| 37/08-1* In view of the number of cases of persons being struck by trains, as shown in section 3.8 of this report (previous similar incidents) and bearing in mind that, in a majority of cases, the railway divides the urban centres which it passes through, it is recommended that the competent authorities take the necessary steps to improve permeability. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0038/08 | 01.09.2008 | 100 Madrid-Hendaya |  | Adif | Renfe Operadora | Collision |
| A light engine passing through Dueñas station collided with a backhoe excavator which had encroached on the track while carrying out trackside work. The driver of the excavator was killed. The accident was caused by the excavator moving into the track clearance envelope after being authorised to do so by the Adif supervisor, such authorisation not being in accordance with the rules set out in the General Traffic Regulations (RGC). |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Adif |  |  |  |
| 38/08-1* Take the necessary steps to ensure that supervisors comply with the General Traffic Regulations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0039/08 | 11.05.2008 | 300 Madrid-Valencia |  | Adif | Renfe Operadora | Person struck |
| A long haul passenger train struck a person at K.P. 202.400 on line 300 Madrid-Chamartín Valencia Estación de Nord. Adif treated this as a case of suicide. The accident was caused by the victim entering a prohibited area. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 39/08-0* Given the nature of the accident there are no recommendations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0040/08 | 04.07.2008 | 270 Portbou-Cerbere-Barcelona-Sant Andreu Comta |  | Adif | Renfe Operadora | At danger signal passed/ near miss |
| A local train entering Sant Celoni station, having passed the distant signal (at amber) entered the section of track where another goods train was stopped at the home signal (at red). At the time of the incident the electrical interlocking system at the station was in the course of being replaced by an electronic system. The accident was caused by a failure to comply with the provisions of Instruction ATO No. 91-CTO No. 92, section 2.3, sub-paragraph 11 ‘Electrical inspection of the new track circuits at the Barcelona end of the line and modification of the track circuits affected by the new arrangement of related distant, home and block signals; until they are re-set electrically the signals affected will give a “stop” indication’. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 3. |  |  | Adif |  |  |  |
| The Technical Investigator endorses the following recommendations formulated by Adif, but makes the additional observation that the creation of a common procedural protocol should be part of the Instruction itself. 40/08-1* The Instructions issued for the purposes of regulating major work on safety installations, particularly where such work has to be compatible with train movements, must clearly specify the single individual, from amongst all the departments involved, who has sole responsibility for communications with the Traffic Manager [Jefe de Circulación] and has full and final responsibility for authorising or suspending the execution of the works. They should also specify the manner in which this official will inform the Traffic Manager of the safety installations affected at each stage, and the operating restrictions which this will entail, so that the Traffic Manager can determine the measures to be taken to ensure the safety of traffic. 40/08-2* Due to the large number of people involved in carrying out work of this kind, there is a high risk of human error and, as has been shown in the past, this can create a threat to the safety of traffic operations, especially in the case of automatic blocking. The work therefore needs to be scheduled into the activities of the maintenance gangs, and consideration might even have to be given to shutting down all traffic on the sections affected by the work and examining possible alternative routes for trains. If this is not possible, telephone blocking will have to be set up to ensure there is only one train on the section. 40/08-3* The various departments involved in carrying out the work will need to draw up a joint operating protocol to ensure perfect coordination of the activities to be undertaken, and to define the pathway for the transmission of orders between the person responsible for communications with the Traffic Manager and the work teams. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0041/08 | 10.09.2008 | 600 Valencia-San Vicente de Calders |  | Adif | Renfe Operadora | Person struck |
| A goods train struck a person at Villareal station. The accident was caused by the victim crossing the track at a prohibited location. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Competent authority |  |  |  |
| 41/08-1* The competent authority should examine the feasibility of fencing off the station precinct and taking other measures to ensure permeability. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0042/08 | 12.09.2008 | 240 San Vicente de Calders-L´Hospitalet de Llobregat |  | Adif | Renfe Operadora | Person struck |
| A goods train struck a person on the boarded crossing at Martorell station in the [ …. text incomplete] The accident was caused by the victim being in a prohibited area as the passenger train was travelling along the line. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 42/08-0* Given the nature of the event, no recommendations are made, since there are signs warning that crossing the track is prohibited, and there is an underpass. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0043/08 | 25.09.2008 | 426 Granada-Fuente Piedra |  | Adif | Renfe Operadora | Person struck |
| A supplementary medium haul train running empty struck a person on the open track at K.P. 14.250 on line 426 Granada-Fuente Piedra. It appears the victim had hearing problems. The accident was caused by the victim being in a prohibited area as the train was travelling along the line. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Competent body |  |  |  |
| 43/08-1* It is recommended that the competent authority installs fixed signs warning of the danger of crossing the tracks and indicating that there is an overpass 200 metres away. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0044/08 | --- | 1031 León-Bilbao |  | Feve | Feve | Collision |
| A railcar, listed as a historic vehicle, struck a set of buffers at Balmaseda Mercancías station after its brakes failed. There were no fatalities. The cause of the accident was the loss of the braking system on the car, causing it to swerve. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Feve |  |  |  |
| 44/08-1* Feve to ensure that historic vehicles travelling on its network comply with the rules relating to maintenance and approval for use, as set out in the Norma Básica de Seguridad del Material [Basic Regulations on Equipment Safety]. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0045/08 | 28.09.2008 | 400 Alcázar de San Juan- Cádiz |  | Adif | Renfe Operadora | Person struck |
| A local passenger train struck a person at K.P. 140.750 on line 400 Alcázar de San Juan- Cádiz. This accident was treated as a suicide by Adif. The accident was caused by the victim being in the middle of the track, in a prohibited location, when the passenger train came along. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 45/08-1* Given the nature of the accident no recommendations are made. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0046/08 | 02.10.2008 | 1031 León-Bilbao |  | Feve | Feve | Person struck |
| A local passenger train struck a person at K.P. 53.725 on line 31 León-Bilbao. The accident occurred at a viaduct over the river Esla between Cistierna and León. The accident was caused by the victim being in a prohibited area. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 2 |  |  | 1. Feve 2. Competent body |  |  |  |
| 46/08-1* It is recommended that, as a provisional measure, the existing “No Entry” notice be repainted and a further three notices be installed, one at each edge of the track and at the access points to the viaduct. 46/08-2* As a definitive measure it is recommended that, since this is an area where persons unconnected with the railway frequently walk, the competent administration should consider erecting a pedestrian walkway linking the two banks of the river, thus eliminating the need for non-railway personnel to use the railway bridge. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0048/08 | 12.10.2008 | 1011 Ferrol-Oviedo |  | Feve | Feve | Person struck |
| A train, carrying out goods train training manoeuvres, struck a person who was walking down the middle of line 3 in the train yard at Pravia station. The accident was caused by an unauthorised person crossing the tracks. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 3 |  |  | Competent body |  |  |  |
| 48/08-1* The competent authority should examine whether the perimeter of this area should be fenced. 48/08-2* Consider whether notices should be erected warning of the danger of crossing the tracks. 48/08-3* The competent authority should consider whether it would be desirable to construct a passageway at a different level to allow crossing from one side of the town to the other. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0049/08 | 13.10.2008 | 300 Madrid-Valencia |  | Adif | Renfe Operadora | Person struck |
| A passenger train struck a person who was crossing line I at the exit from La Encina station. The accident was caused by the victim entering the area where the train was passing through, without being authorised to do so. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Competent body |  |  |  |
| 49/08-1* It is recommended that the competent authority examines the need for fencing off the area. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0050/08 | 15.10.08 | 130 Venta de Baños-Gijón |  | Adif | Renfe Operadora | Person struck |
| A local passenger train struck a person on line 1 at the El Caleyo halt. The accident was caused by the victim entering the area where the train was passing, without being authorised to do so. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Competent body |  |  |  |
| 50/08-1* The competent authority should evaluate the safety of the level crossing between platforms, and thereby determine whether the signage needs to be improved, protection installed or a passageway built at a different level. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0051/08 | 18.10.2008 | 270 Portbou- Cerbere- Barcelona-Sant Andreu Costal |  | Adif | Renfe Operadora | Level crossing |
| A goods train passing over the Category C level crossing at K.P. 247.215 at Figueres station caused a person, who had used the crossing when the barriers were down, to overbalance. The victim, who fell over after the train had passed, did not die immediately but suffered injuries which resulted in their death shortly afterwards. The accident was caused by the victim crossing the level crossing when the barriers were lowered and the light signals and audible warning signals were operating. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 51/08-0* It is not considered necessary to make recommendations because the removal of this level crossing is already planned as part of the work on the Figueres Branch [Variante de Figueres], Barcelona-Portbou line, Phase I . This project was assigned by the Development Ministry to the Sociedad Estatal de Infrastructuras del Transporte Terrestre [Public Company for Land Transport Infrastructures - SEITT] which awarded a contract for the works on 07.07.08 (Official State Gazette No 169 of 14 July 2008). |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0052/08 | 20.10.2008 | 200 Madrid-Barcelona |  | Adif | Renfe Operadora | Level crossing |
| A medium haul passenger train struck a person on the Category C level crossing at Santa María de Huerta station. The accident was caused by the victim crossing the level crossing when the barriers were down and pedestrians were prohibited from crossing. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Railways Directorate (DGF) |  |  |  |
| 52/08-1* Remind the Railways Directorate that it would be desirable to include this level crossing in the Programme for the Removal and Improvement of Level Crossings, when budgets permit. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0053/08 | 23.10.2008 | 100 Madrid-Hendaya |  | Adif | Comsa Rail Transport | At danger signal passed |
| A light engine which was being used for driver training passed the E2 home signal at Villalba de Guadarrama station, which was at red. The incident was the result of insufficient braking which did not adequately reduce the speed, as a consequence of which the locomotive passed the signal beacon at excessive speed. This activated the Automatic Safety Warning and Braking System (ASFA), resulting in an emergency brake application which nevertheless failed to prevent the signal being overrun. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 4 |  |  | 1. Comsa Rail Transport 2. Adif 3. Erion Maintenance Centre 4. Comsa Rail Transport |  |  |  |
| 53/08-1* Ensure that all vehicles which they operate adhere to the schedule set out in the maintenance plan for the various maintenance operations 53/08-2* As an additional element in the monitoring of compliance with the maintenance plans for railway rolling stock, as established by Article 16 of Order FOM/233/2006 of 31 January on the approval of rolling stock, it is recommended that a specific campaign of inspections be carried out to determine the extent to which these plans are being adhered to. 53/08-3* Ensure that the maintenance operations carried out in its workshops meet the levels of consistency specified in the maintenance plans. 53/08-4* It is recommended that the practices employed when driving in adverse braking conditions be improved and made more appropriate to the prevailing conditions. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0054/08 | 24.10.2008 | 600 Valencia- San Vicente de Calders |  | Adif | Renfe Operadora | Derailment |
| A goods train passing through Moncófar station derailed on the points at the exit from the station. The locomotive overturned across the two main tracks and 6 of the 15 wagons were derailed. The driver was slightly injured. The accident was caused by a breakage of the left wheel on the third axle, the first on the second bogie, which initiated a lateral displacement. The right wheel then struck the wing rail, causing flange climb and derailment. The wheel breakage was due to a notch from which a crack then developed and grew progressively as a result of fatigue until the material collapsed. The notch was produced by the jaws of the lathe used to reprofile the wheels after centring. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 5 |  |  | 1-4 Renfe Operadora 5. Rail companies; Maintenance Centres and Adif |  |  |  |
| 54/08-1* Establish inspection procedures and inspection sheets for: Checking for notches or clamping marks caused by lathes. Magnetoscopic inspection of the affected area to ensure there are no cracks. Removal of any such marks. These procedures will apply to all types of wheels which may have been worked on lathes which can create this problem. 54/08-2* Establish an action plan for each series of vehicles affected, indicating the point in the maintenance schedule when each procedure will be applied. Incorporation of these operations into the Maintenance Plans. 54/08-3* Monitor the progress of these actions and the results for each series, with a check being carried out on the results. As the actions progress, and once it is certain that the problem has disappeared, the Maintenance Plans can then be revised to bring them into line with the normal Maintenance Plan for axles of this type. 54/08-4* The systems for clamping wheels during the turning process should be modified to ensure that no marks or notches are created. Quality control at the end of the turning process should include an inspection sheet to ensure that the wheels emerge without notches or marks or, if any are found, that they are removed in accordance with the approved procedure. 54/08-5* All these actions to be notified to the Railway Companies, whose wheels may have been profiled on lathes which produce notches or marks, and these companies must incorporate similar actions in their maintenance plans. They must also check that the turning operations applied to the wheels by the manufacturers do not produce defects of this kind. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0055/08 | 28.10.2008 | 700 Casetas- Intermodal Abando Indalecio Prieto |  | Adif | Adif | At danger signal passed |
| A catenary inspection vehicle passed signal S3/9-15 (displaying a Stop indication) at Casetas station when line 7 was set as the departure route for a goods train. The catenary inspection vehicle was stopped at derailer 22 (a protection device which was set to prevent progress along the track, and which prevented access to line 7). This event was not investigated by the CIAF but the following recommendation was made. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Adif |  |  |  |
| 55/08-1* Since the drivers of track machinery are sometimes unfamiliar with the infrastructure and its installations, given the wide range of works being undertaken across the whole network, it is recommended that there should be a set of rules to ensure that drivers of such vehicles comply with Article 516 of the General Traffic Regulations. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0056/08 | 02.11.2008 | 270 Portbou- Cerbere- Barcelona-Sant Andreu Costal |  | Adif | Renfe Operadora | Person struck |
| A long haul passenger train struck a person crossing the line via a boarded crossing at the Hostalric halt, located at K.P. 170.925. The accident was caused by the victim using the boarded crossing without being aware of the proximity of the train. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1. |  |  | Adif |  |  |  |
| 56/08-1* It is recommended that the boarded crossing be equipped with signage and consideration given to installing whistle boards, if none are present, pending execution of the project work for which a contract has already been awarded for the Tarragona-Barcelona-France Line - Granollers-Maçanet Section - Underpasses between platforms at the Santa María de Palautordera and Hostalric halts. (Official State Gazette No 181 of 28 July 2008). |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0057/08 | 03.11.2008 | 270 Portbou- Cerbere- Barcelona-Sant Andreu Costal |  | Adif | Renfe Operadora | Person struck |
| A local passenger train struck a person crossing the line at Cardedeu station, K.P. 143.105. The accident was caused by the victim entering the area where the train was passing through, without being authorised to do so. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Competent body |  |  |  |
| 57/08-1* It is recommended that the competent authority examines the desirability of fencing off the station car park to prevent pedestrian access from the car park to the line. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0059/08 | 08.11.2008 | 700 Casetas - Intermodal Abando Indalecio Prieto |  | Adif | Renfe Operadora | Person struck |
| A medium haul passenger train struck a person beside the exit signal at Tudela de Navarra station. The accident was caused by the victim entering the area where the train was passing through, without being authorised to do so. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 59/08-0* Given the nature of the event, no recommendation is made. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0060/08 | 09.11.2008 | 522 Manzanares-Ciudad Real |  | Adif | Renfe Operadora | Level crossing |
| A medium haul passenger train struck a vehicle at the Category A level crossing located at K.P. 226.669 between Almagro and the El Campillo halt. The accident was caused by the victim crossing the level crossing without being aware of the presence of the train. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 60/08-0* Given the nature of the event, no recommendation is made. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0061/08 | 10.11.2008 | 810 Vigo- Monforte de Lemos |  | Adif | Renfe Operadora | Level crossing |
| A medium haul passenger train struck a person on a Category C level crossing located at K.P. 2,455 between Monforte de Lemos and Canabal stations. The accident was caused by the victim using the level crossing while the barriers were down. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1. |  |  | Railways Directorate (DGF) |  |  |  |
| 61/08-1* Although the removal of this crossing is planned, until such time as the work begins it is recommended that the Railways Directorate advise the Lugo Provincial Council, through appropriate channels, of its responsibility for ensuring that the fixed road signs meet the requirements of the Order of 2 August 2001 on the removal and protection of level crossings. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0062/08 | 10.11.2008 | 1061 Gijón-Laviana |  | Feve | Feve | At danger signal passed/ near miss |
| A local train passed the stop signal at El Entrego station, resulting in a near collision. The incident was a result of the driver passing the exit signal at the station which was giving a Stop indication (red). This was a breach of Feve’s Train Movement Regulations and Signalling Regulations. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 62/08-0* In view of the nature of the event, no recommendation is made. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0063/08 | 22.11.2008 | 122 Ávila-Salamanca |  | Adif | Renfe Operadora | Person struck |
| A medium haul passenger train struck a person who was riding a motorcycle along the centre of the track in the same direction as the train, at K. P. 104.857. The accident was caused by the victim (who was riding a motorcycle along the centre of the track) entering an area where the train was passing, without authority to do so. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 63/08-0* In view of the nature of the event, no recommendation is made. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0064/08 | 22.11.2008 | 200 Madrid-Barcelona |  | Adif | Renfe Operadora | Person struck |
| A medium haul passenger train struck a person on line G1 at the exit from Alcalá de Henares station, at K.P. 32.800. The accident was caused by the victim entering the area where the train was passing through, without being authorised to do so. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Adif |  |  |  |
| 64/08-1* The broken fence has been repaired and it is recommended that Adif takes the necessary steps to ensure it is maintained. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0065/08 | 26.11.2008 | 700 Caseta-Intermodal Abando Indalecio Prieto |  | Adif | Renfe Operadora | Person struck |
| A medium haul passenger train struck a person who was minding a flock of sheep. The accident occurred on the open track at K.P. 70,200 between Ribaforada and Tudela de Navarra stations. Around 100 sheep were also struck. The accident was caused by the shepherd, who was attempting to get his sheep off the line, entering the track clearance envelope as the train was passing. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 65/08-0* In view of the nature of the event, no recommendation is made. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0066/08 | 29.11.2008 | 300 Madrid- Valencia |  | Adif | Continental Rail | Person struck |
| A goods train struck a workman who was part of the ballast clearance gangs working on the track. The accident occurred when the workman crossed line 1 at K.P. 101.270 at Tembleque station. The accident was caused by the victim entering the track clearance envelope as the train was passing. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1. |  |  | Infrastructure Administrators of the General Interest Rail Network |  |  |  |
| 66/08-1* In view of the accidents occurring in the course of railway works, and the effect of such accidents on rail safety, it is recommended that the Infrastructure Administrators of the General Interest Rail Network establish the necessary procedures - or conduct an analysis of such procedures if they already exist - for the preparation of health and safety studies and plans for railway works and for the authorisation of such works and the monitoring of work in progress, with the aim of ensuring that persons engaged in such work can carry out their activities with minimum risk to persons and to rail traffic. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0070/08 | 14.10.2008 | 500 Madrid- Valencia de Alcántara |  | Adif | Renfe Operadora | Person struck |
| A local train struck a person in the vicinity of points 2 and 4 at the entrance to Leganés station. Adif treated this as a case of suicide. The accident was caused by the victim entering the area where the train was passing through, without being authorised to do so. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 70/08-0* In view of the nature of the event, no recommendation is made. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0071/08 | 13.12.2008 | 276 Maçanet- Massanes- Barcelona- Sagrera |  | Adif | Renfe Operadora | Level crossing |
| A local passenger train struck a man who was crossing the line at the Category C level crossing located at K.P. 48.524 at Calella station. The crossing was closed with all protection activated. The accident was caused by the victim crossing the level crossing when the half barriers were down and the luminous and audible warning signals were activated. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 1 |  |  | Adif |  |  |  |
| 71/08-1* As the constructive proposal for the removal of this crossing is already at the draft stage, it is recommended that the action already in progress to remove it be expedited. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0072/08 | 13.12.2008 | 100 Madrid-Hendaya |  | Adif | Renfe Operadora | Person struck |
| A goods train struck a person walking along the centre of the track with his back to the train. The accident occurred on open track at K.P. 494.315 between Vitoria and Alegría de Álava stations. The accident was caused by the victim entering the track clearance envelope while walking along the left side of the track and attempting to cross the track as the train passed. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 72/08-0* No recommendations are made, since this is a fenced area and there is a pedestrian footbridge and a road underpass in the immediate vicinity. |  |  |  |  |  |  |

| Case No. | Date | Line |  | Administrator | Operator | Type of accident |
| --- | --- | --- | --- | --- | --- | --- |
| 0074/08 | 20.12.2008 | 600 Valencia- San Vicente de Calders |  | Adif | Renfe Operadora | Level crossing |
| A goods train struck a person who was crossing a Category C level crossing at Salou station when the barriers were down. The accident was caused by the victim crossing the level crossing, dodging the closed barriers, at the moment that the train arrived. |  |  |  |  |  |  |
| Number of recommendations |  |  | Recipient of recommendations |  |  |  |
| 0 |  |  |  |  |  |  |
| 74/08-0* Given the circumstances of the accident, and the fact that the removal of this level crossing is already planned, no recommendation is made. |  |  |  |  |  |  |